Inter-Machine
Messaging Standard

for Road Construction Machinery

Draft Version

Version 0.1

Date: 29. January 2003

Contents

1	Scope	3

2	References	3

3	Abbreviations	3

4	Operating principles	3

5	Error handling	4

6	Message Definition	5

7	Configuration	7

1	Scope

This document describes the communication behaviour between road construction machines using any kind of digital communication for data exchange. Machines that comply with this standard are able to communicate with each on construction sites to exchange positioning and process data.

This document was developed by the OSYRIS (Open System for Road Information Support) consortium (www.osyris.org).

2	References

[1]	RFC 1055 “A Nonstandard For Transmission Of IP Datagrams Over Serial Lines: SLIP”

[2]	RFC 919 “Broadcasting Internet Datagrams”

[3]	RFC 3453 “The Use of Forward Error Correction (FEC) in Reliable Multicast”

[4]	RFC 2357 “IETF Criteria for Evaluating Reliable Multicast Transport and Application 
Protocols”

[5]	RFC 1700 “Assigned Numbers”

[6]	IANA (Internet Assigned Numbers Authority) “Internet Protocol V4 Address Space”

3	Abbreviations

OSYRIS

Open System for Road Information Support

4	Operating principles

Equipment that complies to this standard is able to

- connect mobile road machines, primarily rollers and pavers, for the continuous data exchange, to enable distributed functionality, e.g. distributed pass counting or asphalt temperature estimation.

- allow for connection of the site manager, visiting the worksite with a notebook, for the transfer of the design, mission, achieved work and configuration files.

- allow for connection of support engineer, visiting the worksite with a notebook, for remote configuration, diagnostics and possibly remote troubleshooting and debugging.

- allow for connection between the worksite and the site office.

- provide a minimum communication range of 300-1000 m as the machines in the working group stay mostly 200 m or less from each other.

- provide a moving zone of required coverage, as the machine fleet moves at the slow paver pace (~2 m/min),

- provide a minimum bandwidth of at least 20 kbit/s

- support standard network transports (e.g. TCP/IP) and be integrated with OS networking support to provide standard services like file sharing, network browsing etc.

The inter-machine communication is used for the following tasks:

- on-line communication between the machines, so as to exchange relevant process information (continuously),

- communication between the machine and the office: exchange of specification (design and mission) and result information, real-time monitoring of the worksite (few times a day),

- communication between the worksite office and the headquarters: Web access, off-line monitoring of work progress (each few days)

- administrative tasks: Remote configuration and troubleshooting of the system (depending on the need).

The radio connection to be used can be any communication device supporting TCP/IP and providing the above mentioned requirements. It shall be based on widespread Internet networking technology, as defined and maintained by Internet Engineering Task Force (IETF): UDP multicast technology together with IGMP (Internet Group Management Protocol), chosen due to their simplicity, wide support and good performance.

The reliable multicast protocol has the following properties:

- Peer-to-peer operation. Each machine carries whole set of worksite information, improving the robustness of the system.

- Ribbon node is the optimal quantum of the protocol.

- Scalability and internetworking do not pose problems (at most 10 stations in one network)

- Multicast address selection, port numbering, or WLAN network selection can be used to separate fleets.

- The messages require no acknowledgement. In this way the message implosion is avoided. Only negative acknowledgements are sent.

- Reliability is implemented at the process level. The retransmission requests are generated based on the priority queue, newest and longest transmission misses have the highest priority.

- A mechanism for late joining of the fleet needs to be provided. The machines entering the worksite should receive the newest state at high priority.

- Any of the stations may function as soft relay between the stations which are not connected directly.

- UDP provides optionally a capability to verify that the data arrived intact. Alternatively checksum verification at the application level can be used.

- The device independence is provided by Sockets API. However, the algorithm can be easily adapted to any multicast-based communication device.

5	Error handling

The system tries to keep the re-send rate as low as possible. To achieve this, each station keeps record of the missing positions it tries to recover (”list of wishes”) and the requests issued by the other stations, which it may possibly answer (”list of assets”). Internally these two kinds are of items are competing in two priority queues: a missing queue and a request queue. An item with highest priority will be issued first. The priorities change dynamically. For example when the request for missing interval is heard from the other station, its priority is lowered locally and boosted again when no answer has been received. The request priority is high if station is in possession of all the missing data; the request is ignored if the data is fully unavailable. Before adding a request to the missing queue, the station waits at least until the amount of data lost can be estimated. Thus, there is a good chance that somebody else would request and get the same missing data and ”re-send bursts” (requests for the same information) are avoided.

In fact – confirmed by tests – such architecture operates as software relay, taking into account that stations are mobile and the topology of the possible direct connections may change at any time. Let’s suppose two slowly moving stations A and B are too far away to communicate directly (Fig. 1). However, there still is a possibility of exchanging the information via another station C, if it can be simultaneously reached by both A and B (static relay, Fig. 1a). Even if the simultaneous access is not possible, but C moves between the ranges of A and B, the data, however delayed and possibly incomplete, may still be exchanged (Fig. 1b). Both scenarios are relevant to the work site conditions, where the compactors may work on several sections and transfer frequently among them.

a)  b)

Fig.1. Station C as a data relay between stations A and B:  a) static b) dynamic

6	Message definition

Low-level message protocol is based on a connectionless communication scheme, which can be categorised as reliable multicast (Peer-to-All) protocol. It is implemented on top of a unreliable UDP messaging protocol.

Main types of messages are positions (CmPos) and requests messages (CmRequest). The request messages include also confirmations, ping-type of diagnostic messages and alive messages. Other types of messages are position properties (CmProps), immediately following the positions, and text messages (CmMsg).

The messages are sent as follows:

- Positioning messages – during normal operation, whenever a stable position is written into the ribbon database, or as a part of a requested retransmission.

- Re-send request – when a station misses some information, for example when a new compactor enters the work site or when the office requests a report about current situation,

- Confirmation messages – when a station accepts the re-send request, followed by retransmitted positions.

- Alive messages – periodically, when the medium is free and there is no other data to be sent.

- Text messages – displayed on the screen, for operator communication.

- Other diagnostic messages – upon need.

The detailed description of the messages follows the introductory remarks.

- In the medium the UDP messages are preceded with transport specific headers, an IP header (20 bytes) and UDP header (8 byte). Multiple messages may be concatenated in one UDP datagram up to the predefined or maximum allowed datagram length. This is recommended, but not required in order to increase payload-header ratio.

- Messages are immediately preceded by an application specific header (CmHeader) including size and type information.

- The machines are identified by zero-based one-byte indices (Idx), independent from the ID numbers specified in the mission file. Special Idx values are reserved to mean no machine (idNobody = 254) or all machines (idAll = 255).

- The position in the ribbon is the basic quantum of the data exchange. The positions are uniquely identified by the machine id (related one to one with the ribbon number) and a zero-based 32-bit unsigned slot number, which is the sequence number of the position in the ribbon. It should not be confused with the position number, which is the sequence number of position as synchronously calculated by the positioning subsystem. Special value SlotLast = 0xFFFFFFFF is reserved to mean actually last recorded slot.

- Epoch (incremented session number) mechanism has been designed for distinguishing between sessions, but proved unnecessary.

- CRC checking with XOR or CRC16 algorithms may be optionally switched on.

- Tight 1 byte packing has been used. Fillers have been introduced manually in order to increase performance whenever necessary.

CmHeader

| Item | Type | Description |
| --- | --- | --- |
| WCrc | UInt2 | Optional checksum calculated with CRC16 or XOR algorithm |
| WLen | UInt2 | Length of the message including header and checksum |
| WType | UInt2 | Type of message: 1 – CmPos, 2 – CmRequest, 4 - CmMsg, 4- cmEvent, 5 - cmPosProps |
| IdxWho | Byte | Idx of the message sender |
| IdxWhose | Byte | Idx of the message sender |

CmPos

| Item | Type | Description |
| --- | --- | --- |
| dwNumPos | UInt4 | Position number, increased with each position |
| x | Real4 | 3D position relative to worksite center in the East direction [m] |
| y | Real4 | 3D position relative to worksite center in the North direction [m] |
| z | Real4 | 3D absolute position, height [m] |
| Yaw | Real4 | angle position, rotating around vertical axis, [rad, north = 0, west = pi/2] |
| Pitch | Real4 | angle position, rotating around lateral axis, [rad up = positive] |
| Roll | Real4 | angle position, rotating around longitudinal axis, [rad, clockwise = negative] |
| dwFlags | UInt4 | see below |
| dx, dy, dz | Real4 | Position extents (to the right) [m] |
| Speed | Real8 | speed [m/s] |
| Time | Uint64 | Time stamp of the position in100ns units since 1.01.1600 (Win32 FILETIME type) |
| dwNumSlot | UInt4 | Position slot number |
| Epoch | Uint1 | Number of the epoch, incremented at the system restart. Currently ignored. |

Flags

| Flag | Name | Description |
| --- | --- | --- |
| 0000 0000h | POS_INVALID | invalid position |
| 8000 0000h | POS_VALID | valid position |
| 0000 0001h | POS_STOP | machine stopped |
| 0000 0002h | POS_FORWARD | machine moves forward |
| 0000 0004h | POS_BACKWARD | machine moves backward |
| 0000 0080h | POS_VIBRATING | roller in vibrating mode |
| 0000 0020h | POS_ERROR_HEADING | no clear heading of the machine available |
| 0000 0100h | POS_ACCURACY_LOW | low accuracy of the calculated position |
| 0000 0200h | POS_ACCURACY_MED | medium accuracy of the calculated position |
| 0000 0400h | POS_ACCURACY_HIGH | high accuracy of the calculated position |
| 0000 0800h | POS_GPS_SHADOW | Position in GPS shadow zone |

CmPosProps

A table of the following PosProp structures follows optionally the position. Its length identified by the message length.

| Item | Type | Description |
| --- | --- | --- |
| PosPropVal | Union4 | A value of the property as a float, int or string. Currently only floats are used. |
| id | Uint4 | Perfect hash index identifying the name of the property. See props.h. |

CmRequest

| Item | Type | Description |
| --- | --- | --- |
| bType | Uint1 | Type of request: 0 – NAK (negative acknowledgement, normal request), 1 – NCF, NAK confirmation, 2 – ALL, request for all positions, 3 – PING, request for short response, 4 – PONG, short response, 5 – ALIVE – a message with the available slot range |
| From, to | Uint4 | Request range in slot numbers (for NCF, NAK, ALIVE only) |
| X, Y | Real4 | Last position (for PING, PONG only) |

CmMsg

| Item | Type | Description |
| --- | --- | --- |
| szMsg | Char80 | Message to be displayed on the screen |

7	Configuration

The address for multi messages consists of IP address and port number:

- Class-D Mutlicast address is recommended (224.0.0.0 – 239.255.255.255) – address 233.1.1.1 was used for most tests. The multicast addresses require group joining/leaving sequence according to IGMP.

- Broadcast address according to RFC 919 (e.g. A.B.C.255) – not recommended.

- Unicast IP address may be used for testing purposes.

- Port number – arbitrary port from the IANA dynamic and private ports range (49152 - 65535) may be used.

It is recommended to assign the IP addresses of the machines statically in the host file. Moreover the TTL (time-to-live) scoping should be used with the value of 1, restricting the multicast to the subnet.

The configuration parameters are specified in the XML configuration file follows

| Item | Type | Description |
| --- | --- | --- |
| MultiAddr | IP | The address used for sending and receiving of the multi messages. |
| MultiAddr | UInt2 | Port number |
| Beat | UInt | The timer period for multi sends, including request serving, sending of alive messages and flushing of the message queue. |
| MinLen, MaxLen | UInt | The minimum and maximum length of the message queue to be flushed (sent in one datagram). |
| SendPeriod | UInt | Maximum waiting period for the message queue to be flushed. Alive message is sent automatically after this time. Default: 5 secs. |
| TailPos | UInt | Number of last positions to be sent in a response for a request for all. Default: all. |
| TTL | UInt | Time to live (timeout) for the requests and responses in beats. |
| Active | UInt | Mode of operation of the Multi component: a bitmap of values 1 – send positions, 2 – answer requests, 4 – ask requests, 8 – display messages. 7 is default value. |
| MaxRange | Float | A maximum range for a simulation of range-limited transmission. Default value 0. |

currently not defined